MINEGUARD AI

MODULE 2 – SMART MINER SAFETY BAND AND EMERGENCY COMMUNICATION

# 1. Module Overview

Project: AI-Enabled Low-Cost Real-Time Mine Subsidence Monitoring, Prediction and Early Warning System with Smart Miner Safety Band

Module Name: Smart Miner Safety Band and Emergency Communication
Assigned To: Team Member 2

Purpose: Build the software functions that manage miners and wearable bands, receive band telemetry, process fall/SOS events, and support emergency communication and alert broadcasting.

# 2. Smart Band Inputs and Outputs

## Inputs

MPU6050 movement/orientation data.

SOS push button event.

Optional microphone or voice-command event.

Battery level.

Signal/communication status.

## Outputs

Vibration motor command.

Buzzer command.

LED status command.

OLED/display message.

Emergency acknowledgement.

# 3. Miner Management

Create, edit and view miner records.

Assign Miner ID, employee details, mine and zone.

Assign one smart band to a miner.

Track current miner safety/communication status.

# 4. Smart Band Management

Register Band ID and LoRa/device ID.

Link band to miner.

Monitor battery, signal strength and last communication time.

Display online/offline status.

# 5. Fall Detection Workflow

Sudden acceleration
+ Abnormal orientation
+ Possible inactivity
→ Possible Fall
→ Ask/allow miner cancellation
→ No response within configured time
→ Automatic emergency event/SOS

The implementation must allow false-alarm cancellation and event logging.

# 6. SOS System

Receive manual SOS from the smart band.

Create SOS event with Miner ID, Band ID, zone/last known location and timestamp.

Send event to Module 3 central alert/rescue system.

Record acknowledgement and incident status.

# 7. Emergency Communication

## Miner to Control Room

HELP REQUIRED

I AM SAFE

I AM TRAPPED

MEDICAL EMERGENCY

## Control Room to Miner

EVACUATE IMMEDIATELY

MOVE TO SAFE ZONE

STAY CALM

RESCUE TEAM ON THE WAY

# 8. Alert Broadcasting

Broadcast to all miners.

Broadcast to a selected zone.

Send to a specific miner.

Send to a specific band.

Support severity: INFO, WARNING and CRITICAL.

Translate received command into band actions such as vibration, buzzer, LED and display message.

# 9. Suggested APIs

POST /api/miners

GET /api/miners

GET /api/miners/:id

PUT /api/miners/:id

POST /api/bands/data

GET /api/bands

GET /api/bands/:id/status

POST /api/sos

POST /api/fall-events

POST /api/messages

POST /api/bands/broadcast

# 10. Example Band Packet

{
  "band_id": "BAND_001",
  "miner_id": "MINER_001",
  "mine_id": "MINE_001",
  "zone_id": "ZONE_A",
  "timestamp": "2026-08-26T10:30:00Z",
  "battery": 82,
  "signal_strength": 78,
  "motion_status": "NORMAL",
  "fall_detected": false,
  "sos": false
}

# 11. Database Tables

miners

smart_bands

band_status

sos_events

fall_events

emergency_messages

alert_acknowledgements

# 12. Required Screens

Miner List

Miner Details

Smart Band List

Band Detail and Status

SOS Dashboard

Fall Detection Events

Emergency Messaging

Alert Broadcast Page

# 13. Integration Contract with Module 3

Send SOS, fall, low battery and communication-loss events to Module 3.

Receive evacuation and emergency commands from Module 3.

Use common IDs: MINE_ID, ZONE_ID, MINER_ID, BAND_ID, ALERT_ID and INCIDENT_ID.

# 14. Final Deliverables

Miner management system.

Band registration and assignment.

Band telemetry ingestion.

Live band status monitoring.

Fall event processing.

Manual and automatic SOS workflow.

Emergency messaging.

Alert broadcasting.

Integration with Module 3.